Report biomimicry challenge

Urban Heat Island – Cooling the City

# Introduction

## Background & Relevance

The Urban Heat Island (UHI) effect represents a growing environmental and public health challenge, characterized by significantly elevated temperatures in urban environments relative to surrounding rural areas [1]. This effect is worsened by climate change and a high concentration of heat absorbing infrastructure, like cement and asphalt. Conventional mitigation strategies rely heavily on air conditioning, which creates a counterproductive cycle. It increases external temperatures through heat rejection and increases energy consumption, often causing higher greenhouse gas emissions.

To address this problem, the report investigates nature inspired passive cooling mechanisms, also known as biomimicry.

## Design Challenge & Project Contribution

The design challenge lies in developing an efficient, scalable cooling system capable of mitigating the urban heat island effect without relying on high energy consumption.

This project aims to find a solution which bridges the gap between thermal regulation and sustainable urban infrastructure. The goal is to analyze the different heat dissipation methods found in nature, specifically in animals, to find a solution which conforms with the given challenges.

# Challenge & Scoping

## Problem Definition

Conventional cooling methods such as air condition worsen the Urban Heat Island effect by displacing the warm air from inside the building to the surrounding area. This local heat exchange causes the city to warm up more, increasing the need for air condition which ultimately results in a negative feedback loop.

To break this cycle a new alternative and sustainable solution needs to be found.

## Design Question

### Candidate questions considered

Four candidates were formulated in the team and assessed against the criterion that a design question must be neither too broad nor too narrow: specific enough to define an intervention point, open enough to admit different solutions.

| # | Candidate | Assessment |
| --- | --- | --- |
| 1 | How do we prevent heat build-up in urban areas? | Too broad. No place, no time, no intervention point — the equivalent of “How might we end hunger?” |
| 2 | How could we use the energy or heat build-up positively while lowering temperature? | Right idea, but unanchored: no location, and “positively” does not say for what. Refined into candidate 5. |
| 3 | How might we get the heat that Zurich's existing buildings store during the day back out of the city before the next morning, without using energy? | Well formed and highly testable. Needs only small adjustment. |
| 4 | How can we prevent heat waves causing heat islands? | Factually wrong. Heat waves do not cause heat islands. The urban heat island is a permanent consequence of urban surface properties and geometry and exists on ordinary summer days; a heat wave makes it dangerous, it does not create it (Oke, 1982). |

Table 2.1 — Design questions considered, and why four were rejected.

### Selected design question

How might we reduce the urban heat island effect, in cities like Zurich?

The question is stated at the level of the effect, not of the intervention. It names no solution —

no coating, no greening, no geometry, no water — and, unlike candidate 1, it does not pre-commit

to preventing heat build-up, which would exclude the removal half of the problem that Section 2.3

shows to be the half the incumbent measure leaves unsolved. What it deliberately does not do is

narrow the project; that is the job of the rest of this section, and each step is argued rather than

assumed: Section 2.2 fixes the reference city, the reference site and the place the effect must

appear, which makes the façade the system of interest; Section 2.4 fixes what the design may

touch; Section 2.5 states what it must do; and the KPIs in Section 2.7 fix what “reduce” has to

mean in numbers.

One property is carried forward from candidate 3 and is not visible in the wording of the

question, so it is stated here explicitly: the response must be self-regulating, its cooling power

rising with the heat load instead of being fixed. This is a decision of the team rather than a

demand of the question, and it is the decision that separates the project from the incumbent

measure. A bright surface has a fixed optical response: it reflects the same fraction of radiation

at 20 ◦C as at 40 ◦C, and it does nothing at all once the sun has set. A self-regulating surface

behaves like a biological one — a leaf’s response is driven by the same solar energy that heats

the leaf, so the cooling effort scales with the load, without a controller, a sensor or a pump. It

becomes function F5 and is measured by KPI 1.

That decision also resolves the team’s initial difficulty of having two systems of interest

(“reduce heat” and “prevent heat”). These are not two systems. They are two functions of

one system: prevention acts during the day window, removal during the night window, and the

self-regulating link connects them.

## Context and Constraints

Reference city: Zürich. All context values, benchmarks and engineering checks in this report refer to the City of Zurich. Fixing one reference city is what makes the constraints and the KPI targets checkable instead of generic. Zürich was chosen for four reasons:

- The city has an adopted municipal heat-mitigation strategy, the Fachplanung Hitzeminderung (Stadt Zürich, 2020), with an implementation agenda, a guidance leaflet for building owners and three thematic maps. The project therefore has a real client, a documented baseline and published target values instead of invented ones.

- The city publishes an open climate analysis and open geodata, so the reference site can be selected from measured heat-load data instead of intuition.

- MeteoSwiss operates long-term stations in and around the city, which supplies the boundary conditions (G, Ta, wind speed, humidity) for the energy balance in Section 3.2.

- Its building stock and morphology are representative of the Swiss Mittelland, so results transfer to comparable Swiss cities.

Reference site and users. We adopt the city's own model area Modellierungsgebiet 03 - Geschlossene Randbebauung (closed perimeter block) as the reference site (Stadt Zürich, 2020, pp. 148–151). It consists of two outward-closed blocks of five stories plus a roof structure enclosing private courtyards, in a dense district close to the center; the street space is sealed from façade to façade and only isolated trees provide shade. The city's own assessment is that the reduced air exchange and the lack of cold-air supply make this structural type overheat comparatively quickly. Choosing this site has a methodical advantage: the Fachplanung already publishes modelled baseline and climate-optimized results for exactly this area, so our concept can be compared against a documented reference rather than against our own assumptions.

The users are the residents of the upper floors, who carry the strongest heat load and usually have no mechanical cooling; the pedestrians in the street canyon, who receive radiation from the façade at eye level; the building owner, who decides on and pays for the retrofit; and the trades who install and later maintain the system.

Constraints. The table below separates hard constraints (a violation disqualifies a concept) from soft ones (they weight the evaluation in Section 3.3). The constraints on installation and maintenance come from the team's observation that the people who mount and service the system are part of the context; they are recorded here rather than as subsystems, because the system of interest contains components, not people.

| Type | Class | Constraint |
| --- | --- | --- |
| Energy | hard | Limit auxiliary electrical energy for the cooling function. |
| Water | hard | No water waste. |
| Existing building | soft | Preferable, but not neccessary. |
| Townscape | hard | The result must be acceptable in the Zurich townscape. Buildings may be listed; heritage protection, building law and neighbour rights can restrict visible changes (Stadt Zürich, 2020, p. 128). |
| Glare | hard | No specular glare towards streets, neighbouring flats or traffic. The city names glare as the main objection to bright envelopes, which favours diffuse over mirror-like surfaces. |
| Climate | hard | Must survive the Swiss annual cycle: frost to about −10 °C, freeze–thaw cycles, driving rain, UV, and periods of several dry weeks in summer. |
| Winter | hard | Must not appreciably increase heating demand. A high solar reflectance also reduces useful winter solar gains; this trade-off must be quantified, not ignored. |
| Installation | soft | Must be installable by ordinary trades on an occupied building, ideally without full scaffolding. |
| Maintenance | soft | Must remain functional with little maintenance. Soiling and the resulting cleaning cost are named by the city as a specific weakness of high-albedo envelopes. Service life target ≥ 20 years. |
| Solar energy | soft | Should not unnecessarily compromise roof- or façade-integrated photovoltaics; the city notes that bright envelopes reduce PV yield. |
| Structural | soft | Limited additional dead load and build-up thickness on an existing envelope. |
| Embodied emissions | soft | The greenhouse gas embodied in the added build-up must be repaid by the avoided cooling demand within the service life, measured in kg CO₂-eq per m². |
| Economic | soft | The city expects higher cost for high-albedo envelopes. Target: cost per square metre in the range of a normal refurbishment. |

Table 2.3 — Context conditions and constraints.

## System View and Project Boundary

The hierarchy below prevents tunnel vision, and the boundary drawn after it prevents scope explosion. Together they reveal interfaces, functions, constraints and KPIs.

| Level | In this project |
| --- | --- |
| Super-system | The city and its existing infrastructure. |
| System of interest | The heat-regulating outer layer of a building. |
| Parallel systems | Stormwater and sewer network · street and pavement network · power grid and building energy system · residents, transport and public transit |
| Subsystems | Outer surface · storage and insulation layer · materials · water path · attachment to the substrate |

Table 2.5 — System hierarchy.

| IN the project | OUT of the project |
| --- | --- |
| Prevent heat buildup and remove heat from the building façade. | Redesign of the building itself. Reuse of the stored energy. No internal cooling. |

Table 2.7 — Project boundary: what is deliberately in and out of the project.

## Functions and Functional Diagram

They are stated at the scale of the designed object, not at the scale of the city: the design regulates heat at the surface it occupies, and the city-scale effect is the goal, not the function — in the same way that the function of polar bear fur is to insulate, not to make the Arctic habitable.

| ID | Function (verb) | Explanation / taxonomy link |
| --- | --- | --- |
| F1 | Reflect short-wave radiation | Prevention - Avoid absorbing solar energy (Visible & NIR light). |
| F2 | Shade a surface | Prevention - Reduce direct light exposure on structural surfaces. |
| F3 | Dissipate / Transfer heat | Removal - Transfer thermal energy via transmission or thermal radiation. |
| F4 | Evaporate liquid | Removal - Heat extraction using moisture or water |
| F5 | Resist degradation | Supporting - Maintain long-term durability against UV, rain, and frost. |

Table 2.8 — Function list. Main function, sub-functions and supporting functions.

## Biologized Questions

- How does nature use the heat it receives to get rid of that same heat?

- How does nature release excess heat to the night sky?

- How does nature reflect intense solar radiation without pigments that bleach, and without a mirror finish?

- How does nature shade a surface even in the desert?

- How does nature keep a functional surface clean without cleaning it?

## Performance Criteria and KPIs

The KPIs are derived from the functions, interfaces and constraints identified above, not invented independently of them. They are used twice: to guide the engineering checks in Section 3.2 and to compare the concepts in Section 3.3. The reference case is an existing Zurich façade with a solar reflectance of about 0.3, the value the city uses for un-greened façades.

| # | Function | KPI | Unit | Target / benchmark |
| --- | --- | --- | --- | --- |
| 1 | F5 Couple response to load | Self-regulation gradient ∂Pcool/∂Ts | W/m²K | Must be clearly positive and larger than for a fixed high-albedo surface. The defining criterion of this project — it is the quantity the design question asks for. |
| 2 | F1 Reflect short-wave radiation | Solar reflectance αsol (300–2500 nm) | — | ≥ 0.80, the threshold the city defines for “high albedo”. Baseline 0.3. Ultra-white laboratory paints reach ≈ 0.98 (Li et al., 2021). |
| 3 | F0 Regulate heat | Peak surface temperature reduction ΔTs vs reference, clear sky, 14:00 | K | ≥ 15 K, to be confirmed by calculation or measurement before claiming it. |
| 4 | F0 (day, people) | PET reduction at 2 m above ground, 14:00 | K | Beat the incumbent. Since the system of interest is the façade, the binding figure is HA 11's −1.0 K median for a bright façade; the −1.2 K for a bright roof is context. This is the metric the client uses. |
| 5 | F2, F4 (evening / night) | Net cooling power at 02:00, clear sky; and heat discharged over the evening and night windows, 18:00–06:00 | W/m²; Wh/m² | ≥ 40 W/m² instantaneous (Raman et al., 2014; Zhai et al., 2017), and a night-time air temperature effect better than the −0.1 K of HA 11. |
| 7 | F6 Durability | Retention of the cooling function after soiling and weathering | % of initial | ≥ 90 % after three years. Soiling is the weakness the city names for high-albedo envelopes; aged cool roofs commonly lose a significant fraction of their initial reflectance (Santamouris, 2014). |

Table 2.9 — Performance criteria. Function → KPI → unit → target.

The reset condition. KPI 1 and KPI 5 are evaluated per day, but over the five-day design case a further condition applies: the surface temperature and the state of charge of the heat store at 06:00 must not drift upwards from one day to the next. A concept that cools well on the first day but saturates by the third fails the design case even if every single-day KPI is met.

Important distinction. KPI 3 measures a surface temperature, KPI 4 an effect on people. A high-reflectance surface can be 20 K cooler than a dark one while the effect on the air temperature of the district is smaller by roughly two orders of magnitude (Santamouris, 2014; Akbari et al., 2009). Conflating the two is the most common error in urban-heat-island projects, so the two are reported separately throughout this report.

Note on the targets. The targets above are aspirational benchmarks set during scoping. They are hypotheses, not results. Each is checked in Section 3.2 and either confirmed, revised or explicitly declared unverified.

# Bio-Inspired Design Process

This section delivers the biology and the abstraction, and hands a set of transferable principles to the concept development in Section 3.2.

## Discovering & Abstracting

### Research Approach and Biologized Questions

The biological research was derived from the solution-neutral functions identified during the scoping phase. Instead of searching directly for architectural solutions, the functions were translated into biologized questions. This function-based approach reduces the risk of prematurely selecting a specific technical solution and allows biological strategies from different organisms and systems to be compared on a common functional basis.

The search focused on four principal questions. First, how does nature prevent excessive solar heat gain through reflection or self-shading? Second, how does nature transfer, release or temporarily buffer heat? Third, how does nature capture, store and regulate limited water resources? Finally, how does nature adjust the intensity of a cooling response to changing thermal loads without relying on an external controller? These questions correspond to the prevention, heat-removal, water-management and self-regulation functions identified during scoping.

Natural models were subsequently compared according to their underlying mechanism rather than their external appearance. Particular attention was given to passive strategies that could potentially operate with low external energy input and that could be transferred to façades, roofs or other elements of the urban envelope.

### African Elephant: Ears

Biological mechanism. African elephants (Loxodonta africana) use their large, highly vascularised ears as heat-exchange surfaces. Warm blood is transported from the body core into the thin ear tissue, where heat is transferred to the surrounding air. Ear movement increases airflow across the surface and can further enhance convective heat transfer. AskNature identifies radiation from blood vessels and generated air currents as the principal cooling mechanisms (AskNature, n.d.), while the University of Notre Dame also describes the ears as large heat sinks with controllable blood flow (University of Notre Dame, 2021).

Functional abstraction. Transfer thermal energy from a heat-generating region to a large, exposed exchange surface and increase airflow across this surface.

Design principle for urban cooling. Use dedicated heat-rejection surfaces in locations with favourable airflow to transfer and dissipate heat from urban structures without active refrigeration.

### Elephant Skin: Sparse Hairs

Biological mechanism. African elephants have a very low density of body hair. At this density, individual hairs do not form an insulating layer but act as small heat-transfer fins. The hairs increase the effective surface area of the skin and thereby support heat exchange with the surrounding air. The University of Notre Dame reports that elephant hair density lies well below the density at which insulation becomes dominant, and puts the additional heat loss at up to 23 % (University of Notre Dame, 2021). That figure traces to a university news article rather than to a study, which is recorded as an open item in Appendix A.

Functional abstraction. Increase heat-transfer area by distributing sparse protrusions across a warm surface.

Design principle for urban cooling. Use small surface structures on façades or other heated urban surfaces to enhance passive convective heat transfer without substantially increasing material use.

### Jackrabbit Ears and Elephant Thermal Windows

Biological mechanism. Black-tailed jackrabbits regulate heat loss by altering blood flow through the dense vascular network in their ears. Vasodilation transports more warm blood from the body core towards the exposed ear surface when surrounding conditions favour heat loss; at an air temperature of about 30 °C the ears shed all excess heat, and the strategy exists to avoid spending water. Elephants use a comparable strategy through vascularised regions close to the skin surface, described as thermal windows, which can be supplied with more or less blood depending on thermal conditions (AskNature, n.d.).

Functional abstraction. Regulate cooling by controlling the transport of heat towards selected exchange surfaces according to environmental conditions.

Design principle for urban cooling. Create distributed heat-rejection zones whose thermal connection to the main structure changes according to cooling demand and external conditions.

### Cactus Ribs

Biological mechanism. Many cacti have ribbed surfaces consisting of repeated peaks and troughs. The peaks shade parts of the troughs and therefore reduce direct solar heat gain. Air within the shaded troughs remains cooler, absorbs heat from the cactus and rises as it warms. The ribbed geometry also increases surface area and disrupts airflow around the plant (AskNature, n.d.).

Functional abstraction. Use folded surface geometry to combine self-shading, increased heat-transfer area and controlled interaction with airflow.

Design principle for urban cooling. Integrate ribbed or folded geometries into urban surfaces to reduce direct solar exposure while simultaneously promoting passive heat dissipation.

### Desert Snail Shell

Biological mechanism. The desert snail Sphincterochila boissieri limits heat gain through a highly reflective shell and internal insulating air spaces. AskNature reports approximately 90 % reflectance in the visible spectrum and about 95 % in the near-infrared range (AskNature, n.d.). The snail also withdraws into the upper parts of its shell, while air-filled spaces and limited contact with the hot substrate reduce conductive heat transfer.

Functional abstraction. Reduce incoming radiant energy and interrupt conductive heat transfer using reflective surfaces and stationary air layers.

Design principle for urban cooling. Combine highly reflective external surfaces with insulating or ventilated air cavities to limit solar heat storage in roofs and façades.

### Morpho Butterfly Wing

Biological mechanism. Morpho butterflies generate their characteristic colour using transparent chitin-and-air structures at micro- and nanoscale. Ridges and cross-ribs interact with incoming light through diffraction and interference, causing selected wavelengths to be strongly reflected. The colour therefore results primarily from physical structure rather than pigmentation (AskNature, n.d.).

Functional abstraction. Control the spectral interaction between a surface and incoming radiation through micro- and nanoscale geometry.

Design principle for urban cooling. Engineer structured surfaces that selectively manipulate incident solar radiation while maintaining desired optical properties. The limitation must be stated with the principle: the Morpho demonstrates structural colour rather than near-infrared heat rejection, and that cooling application requires separate technical verification.

### Leaf Stomata

Biological mechanism. Plant stomata are microscopic pores surrounded by pairs of guard cells. Environmental and chemical signals alter the concentration of solutes within these cells, causing osmotic water transport and changes in cell turgor. The guard cells consequently bend to open or close the pore. This mechanism regulates gas exchange while limiting excessive water loss. Transpiration through open stomata cools leaf surfaces by 11–17 K, and the evaporation itself draws the water up, with no pump anywhere (Biomimicry 3.8 & HOK, 2013, pp. 62–65).

Functional abstraction. Use environmental signals to passively vary the size of an opening and thereby regulate material exchange.

Design principle for urban cooling. Develop responsive openings that regulate ventilation or evaporative cooling according to temperature, humidity or water availability without continuous active control.

### Camel Fur and Sweat

Biological mechanism. Dromedary camels combine insulation with evaporative cooling. Sweat glands release water that removes body heat during evaporation, while the thick fur reduces incoming heat. According to AskNature, the fur limits radiative heat gain through its light colour, conductive transfer through trapped air and convective transfer by restricting air movement close to the skin; in the technical reproduction of that arrangement the pair held a sample 7 K below ambient for 200 h, five times longer than the wet layer alone (AskNature, n.d.).

Functional abstraction. Reduce the incoming thermal load before using a limited consumable resource for additional cooling.

Design principle for urban cooling. Combine shading or thermal insulation with controlled evaporative cooling so that less water is required to achieve the desired cooling effect.

### Pine Cone Scales

Biological mechanism. Pine cone scales respond to humidity despite containing no muscles or active control system. Their layered structure contains materials with different swelling behaviour. When moisture is absorbed, one layer expands more than the other, causing the scale to bend and close. As the scale dries, it returns towards its open state. The movement is reversible and the cone is dead tissue, moved entirely by the weather (AskNature, n.d.).

Functional abstraction. Convert an environmental change directly into mechanical movement through differential material expansion.

Design principle for urban cooling. Use environmentally responsive material layers to create self-actuating shading or ventilation elements that operate without motors or electronic control.

### Honeybee Fanning Thresholds

Biological mechanism. Western honeybee colonies regulate nest temperature through the combined behaviour of many workers. Individual bees have different temperature-response thresholds, partly resulting from genetic diversity: a colony has one queen but many fathers, so it contains patrilines with different thresholds. As nest temperature increases, progressively more bees begin ventilating the hive. This distributed response prevents excessive system-wide reactions to small temperature fluctuations and stabilises the nest below about 36 °C with no central control (AskNature, n.d.).

Functional abstraction. Distribute different activation thresholds across multiple independent elements to create a gradual system-level response.

Design principle for urban cooling. Use multiple passive cooling elements with different trigger conditions so that cooling capacity increases progressively with thermal load instead of operating through a single on/off threshold.

### Lichen Surface

Biological mechanism. The lichen Lecanora conizaeoides combines a rough external surface with hydrophobic micropores. Water remains primarily on the upper surface structures and is shed rather than blocking the pores. The hydrophobic pores therefore remain available for gas exchange even under wet conditions (Biomimicry 3.8 & HOK, 2013, pp. 41–44).

Functional abstraction. Use hierarchical surface roughness and selective wettability to separate liquid transport from gas transport across the same surface.

Design principle for urban cooling. Create water-repellent but permeable façade surfaces that protect ventilation or gas-exchange pathways from blockage during rain.

### Mechanisms in depth

Although the biological models originate from different organisms and scales, their mechanisms can be reduced to several recurring design principles. The first is prevention before removal: cactus ribs, desert snail shells and camel fur reduce the thermal load before additional cooling is required. The second is geometry-driven heat transfer: elephant ears, sparse hairs and cactus ribs use surface area, exposure and airflow rather than active refrigeration.

A third recurring principle is distributed passive regulation. Jackrabbit blood flow, stomatal opening, pine-cone deformation and honeybee fanning all adjust their response according to changing environmental conditions without a conventional central controller. The fourth principle concerns resource limitation. The camel and stomatal strategies indicate that evaporative cooling should not be considered independently of water availability. Cooling demand should first be reduced and the remaining water resource should then be metered according to thermal demand.

These abstractions are more relevant to engineering transfer than the literal biological forms. For example, a biomimetic application does not require an elephant-ear-shaped façade or a cactus-shaped building. The transferable information lies in increased exchange area, disrupted boundary layers, selective heat transport, self-shading geometry or environment-responsive actuation. This corresponds to the course requirement that abstraction focus on mechanism rather than copying the organism.

### Abstraction into design principles

Table 3.3 keeps the three columns strictly separate, as the method requires: what was observed, what the observation abstracts to, and what it might become technically. Only the third column contains anything that could be built, and nothing in it has been selected yet — that is the work of Section 3.2.

| Biological observation | Abstracted principle | Possible technical interpretation |
| --- | --- | --- |
| The snail’s shell reflects strongly and an air space separates it from the hot substrate | Reflect first, then decouple whatever still gets through with a layer of air | Reflective outer skin over a ventilated cavity; structural white instead of a pigment |
| Morpho colour comes from layer geometry, not pigment, and the disordered structure stays diffuse | Separate what a surface looks like from what it does with heat | Structurally coloured or near-infrared-selective finish that keeps the existing façade colour |
| Cactus peaks shade the troughs; cooler trough air absorbs heat and rises out | Use folded geometry for self-shading, more emitting area and better airflow | Ribbed or profiled façade and paving relief |
| Sparse hairs reach through the still air layer clinging to the skin | Break the boundary layer so moving air reaches the surface | Millimetre-scale texture on façade and pavement |
| The jackrabbit varies blood flow to a thin radiator; the elephant opens patches | Regulate by varying the thermal connection between the store and a radiating surface, and place that radiator only where conditions favour loss | Pumpless density-driven liquid loop from the canyon face to a parapet-level radiator with a good sky view |
| Guard cells open and close the pore in response to heat and dryness | Build the control into the opening, so the escape route widens with the load | Load-driven valve or wick, rather than a controlled one |
| Camel fur insulates over the evaporating skin | Cut the load first, then spend a little water on the remainder, and cover the wet layer so the reserve lasts | Absorbent layer beneath a vapour-permeable cover |
| Worker bees each switch at a different temperature | Spread many binary elements across a range of trigger points and the sum responds continuously | Any switching element built as many small units with a deliberate spread of switching temperatures |
| Pine cone scales bend because two bonded layers swell differently | Make moving parts from two layers that respond differently and let the environment actuate them | Bilayer shading element; hygroscopic or thermal actuation, no motor |
| The sea star charges on cold water sized by the previous hot exposure | Charge the store during the cool phase, sized by the load of the previous cycle | Night-charged store, with an explicit reset condition across the five-day case |
| Lichen roughness at several scales sheds water while staying breathable | Use geometry rather than chemistry to shed water and stay clean | Multi-scale textured finish; no biocide in the film |

Table 3.3 — From biological observation to transferable design principle. The third column lists possibilities, not decisions.

Cross-model abstraction. Although the biological models originate from different organisms and scales, their mechanisms reduce to several recurring principles. The first is prevention before removal: cactus ribs, desert snail shells and camel fur reduce the thermal load before additional cooling is required. The second is geometry-driven heat transfer: elephant ears, sparse hairs and cactus ribs use surface area, exposure and airflow rather than active refrigeration. The third is distributed passive regulation: jackrabbit blood flow, stomatal opening, pine-cone deformation and honeybee fanning all adjust their response to changing conditions without a central controller. The fourth concerns resource limitation: the camel and stomatal strategies indicate that evaporative cooling cannot be considered independently of water availability — cooling demand should first be reduced and the remaining water metered according to thermal demand.

These abstractions are more relevant to engineering transfer than the literal biological forms. A biomimetic application does not require an elephant-ear-shaped façade or a cactus-shaped building. The transferable information lies in increased exchange area, disrupted boundary layers, selective heat transport, self-shading geometry or environment-responsive actuation.

One principle, found three times. The jackrabbit and the elephant regulate by how much area is active, the bees by how many units have switched, and a bilayer shading element by how far it has deployed. All three modulate how many elements act rather than how strongly each one acts, and all three arrive at a graded response without a controller. That convergence from three unrelated organisms is the strongest abstraction this project has produced, and it is the one carried into every concept in Section 3.2.

### Life’s Principles evident in the selected strategies

The principles below are the ones the selected models actually exhibit; Section 4 assesses how well our final concept applies them, which is a different question.

Be locally attuned and responsive — present in every model that couples its response to the local load, and named by the checklist in the form our F9 requires: the signal, the antenna that detects it and the response must match (Baumeister et al., 2014).

Be resource efficient (material and energy), sub-principle use low energy processes — none of the selected mechanisms consumes energy; the load itself drives them.

Adapt to changing conditions, sub-principles incorporate diversity and maintain integrity through self-renewal — Encelia and the pine cone for the first, the lichen for the second.

Use life-friendly chemistry — Cyphochilus achieves with geometry what industry achieves with titanium dioxide. The lichen is the more careful case: its water repellency does involve a compound, but that compound works with a multi-scale geometry rather than instead of it, and nothing is released to do the job.

## Creating: Engineering Concept Development

### Ideation

Ideation started from the abstracted principles rather than from free sketching. Eight directions were generated in a Crazy 8s round, and the rule was applied strictly: vary the mechanism, not the shape. Eight versions of the same louvre would have been one idea drawn eight times.

| # | Direction | Mechanism varied | From which model |
| --- | --- | --- | --- |
| 1 | Structural white | scattering without pigment | Cyphochilus · desert snail |
| 2 | Spectral split | keep the visible colour, reject the near infrared | Morpho wing |
| 3 | Self-shading relief | folded geometry shades itself | cactus ribs |
| 4 | Boundary-layer texture | millimetre roughness raises convection | elephant skin hairs |
| 5 | Transport to a better surface | pumpless loop to a roof radiator | elephant ear · jackrabbit ear |
| 6 | Night-charged store | charge in the cool phase, sized by the previous cycle | sea star |
| 7 | Load-driven aperture | an opening that widens as it heats | leaf stomata |
| 8 | Passive bilayer actuator | deploys with heat or moisture, no motor | pine cone scales |

Table 3.4 — The Crazy 8s round. Each direction is a different physical mechanism, and each is traceable to a model from Section 3.1.

Directions 1 and 2 act on what is absorbed, 3 and 4 on the surface geometry, 5 to 7 on removal and regulation, 8 on actuation. That spread is what the exercise is for: it shows the field was wider than the two concepts that survived.

[TODO: Add a photo or scan of the sheets to Appendix C. The eight directions above are the documented output of the round; the sheets are the artefact that proves it happened as a team exercise.]

### Morphological matrix

The method is function-based concept generation (Nagel et al., 2010): the rows of the matrix are functions, the columns are alternative principles that serve them, and a concept is a path across the rows. The rows are chosen, not inherited. A row belongs in the matrix only where alternative principles would lead to a meaningfully different concept, which leaves four.

| Function | Option 1 | Option 2 | Option 3 |
| --- | --- | --- | --- |
| Reduce heat input | Bright or near-infrared-selective finish (B) | Ribbed, self-shading geometry (A) | Deployable shading element |
| Move heat away | Broadband emitter with a good sky view (A) | Boundary-layer texture for convection (B) | Liquid loop to a roof radiator (A) |
| Store and buffer | Thermal mass in the build-up | Thermal mass carried in a liquid (A) | No added storage; decoupling only (B) |
| Couple response to load | Switching material, spread thresholds | Buoyancy — the driving head grows with ΔT (A) | Convection — the flux grows with ΔT (B) |

Table 3.5 — Morphological matrix. (A) marks the path taken by the water façade, (B) the path taken by the roughness spray.

Three combinations were read off the matrix and checked for physical compatibility. Water façade (A): ribbed skin, thermal mass in a liquid, buoyancy-driven loop to a roof radiator. Roughness spray (B): bright finish plus boundary-layer texture, no store. Switching coating: a thermochromic finish that darkens in winter and brightens in summer — dropped, because it acts only on what is absorbed and has no effect in the night window, which Section 2 identifies as the larger half of the problem. It would have reproduced the incumbent measure with extra chemistry.

The two surviving paths share no cell. That is the test of whether a pair is genuinely distinct rather than two stylings of one idea, and it is also why the switching coating was the one dropped: its path would have overlapped B in three of the four rows.

How biology was balanced against feasibility. Nothing in either concept reproduces a biological form. The elephant's ear contributes the arrangement — absorb in one place, release in another, and let the transport track the load — not a shape. The pipework itself is conventional building services, and only its pumplessness is biomimetic. The cactus contributes a folded profile, manufactured as extruded relief rather than a plant silhouette. The elephant's hair contributes a length scale relative to the boundary layer, which becomes a sprayed aggregate, not a bristle.

### Concept A — the water façade

What it is. A new-build façade whose outer skin is a ribbed, light-coloured panel and whose build-up carries a closed water loop. The ribs shade the troughs between them, as a cactus does, so part of the surface is never in direct sun. Behind the panel, wide-bore risers absorb what the surface does take up. Water warmed in the risers becomes less dense and rises on its own; cooled water in the return line sinks. At the top the loop opens into a thin, wide plate at roof level, where the water is spread over the largest area the roof allows and gives its heat to the night sky and to the air above the building. Nothing is pumped, nothing is controlled, and no water is consumed — the loop is closed.

Functional chain. Prevention at the ribbed surface reduces what is absorbed; the water stores it instead of the masonry; the roof plate releases it. The load-coupling the design question asks for is not an added device but the physics of the loop itself: the warmer the façade, the larger the density difference between riser and return, the larger the driving head, and the faster the circulation. The response scales with the load without a sensor.

| Component | Origin | What was transferred |
| --- | --- | --- |
| Ribbed outer panel | Cactus rib (AskNature, n.d.) | Folded geometry that shades itself and enlarges the emitting area. The profile is extruded, not plant-shaped |
| Riser and return | Elephant ear, jackrabbit ear (AskNature, n.d.) | The arrangement: absorb in one place, release in another, and let the transport rate follow the load. The pipework is conventional |
| Buoyancy drive | — | Ordinary thermosiphon physics. The biology suggested pumplessness; it did not supply the mechanism |
| Roof plate | Elephant ear | Maximum thin area at the place with the best view of the sink |
| Charging behaviour | Sea star (AskNature, n.d.) | Charge during the cool phase, sized by the previous cycle; the reset condition of Section 2 comes from here |

Table 3.6 — Concept A: which parts are transferred from biology and which are ordinary engineering.

Materials and parameters. [TODO: Fix the panel material and its solar reflectance and emittance; the pipe material and its corrosion and freeze behaviour; the loop volume; and the roof plate construction. The bore follows from Engineering check 1 and is not free.]

### Concept B — the roughness spray

What it is. A sprayable coating carrying a millimetre-scale aggregate, applied to an existing bright façade. The particles stand proud of the surface far enough to reach through the layer of still air that clings to any smooth wall, so that moving canyon air touches the surface instead of gliding over an insulating film. It adds no moving parts, no water and no construction; it is applied by ordinary trades, is close to invisible from the street, and is renewed by spraying again.

Functional chain. Reflection at the bright finish, convection at the texture. There is no store and no radiative path beyond what the wall already has: B deliberately buys one function well rather than four badly. Its load-coupling comes from the convective term itself — the flux is proportional to the surface-to-air difference, so the response rises with the load, linearly rather than steeply.

Where the biology enters. The elephant's sparse hairs sit below the density at which hair insulates, so each acts as a fin through the boundary layer rather than as part of a blanket; the reported effect is an additional heat loss of up to 23 % (University of Notre Dame, 2021). The transferred quantity is a length scale relative to the boundary layer, not a bristle. The lichen contributes the second requirement: multi-scale roughness that sheds water and stays clean without a biocide (Biomimicry 3.8 & HOK, 2013, pp. 41–44). That this is manufacturable rather than speculative is established by prior art — StoColor Lotusan, a micro-structured superhydrophobic façade paint modelled on the lotus leaf, has been sold since 1999 and is stated by its manufacturer to resist algae and fungal growth without biocide film protection (Sto SE & Co. KGaA, 1999).

The obvious objection. Roughness can also thicken the still layer instead of piercing it, if the elements are too short or too dense. This is the concept's decisive assumption and it is critical question 4 below. [TODO: Fix the particle size, the areal density and the binder, and state the expected convective coefficient before and after.]

### Engineering Canvas and critical questions

| Field | A — water façade | B — roughness spray |
| --- | --- | --- |
| Function | Reduce input, store, move heat away, couple response to load | Reduce input, hand heat to the air, couple response to load |
| Biological mechanism | Ear-based transport; cactus rib; sea-star charging | Sparse hair as a boundary-layer fin; lichen roughness |
| Abstracted principle | Move heat to where release is easy; let the load drive the transport | Break the still layer so moving air reaches the surface |
| Technical principle | Buoyancy-driven closed water loop to a roof radiator | Sprayed millimetre aggregate on a bright finish |
| Architecture | Ribbed panel → riser → roof plate → return | Substrate → primer → textured top coat |
| Inputs / outputs | In: solar gain. Out: heat to sky and air. No mass crosses the boundary | In: solar gain. Out: heat to canyon air |
| Requirements / KPIs | All six criteria; build-up thickness and dead load | Reflectance, load coupling, durability; installability |
| Assumptions | The buoyancy head is enough; the roof area is enough; the loop survives winter | The texture pierces rather than thickens the boundary layer |
| Uncertainties | Freezing; roof area; flow distribution across parallel risers | Soiling; abrasion; whether the gain survives at low wind |
| Next check | Checks 1 and 2 below | [TODO: wind-tunnel or two-plate comparison] |

Table 3.7 — Engineering Canvas for both concepts. Its output is the list of critical questions below.

A critical question names a quantity, a threshold and a scale. Four were derived from the canvas, and they decide between the concepts rather than merely describing them.

1. Does the loop drive itself? Is the buoyancy head over the available building height enough to move the required flux through a pipe bore that still fits the allowed build-up thickness? Checked below.

2. Where does the heat actually go? Is the roof area large enough to release what the façade collects? Checked below — and the answer changed the design.

3. Does the loop survive a Swiss winter? Frost protection to about −10 °C without auxiliary energy is not yet designed. [TODO: open]

4. Does the texture pierce the boundary layer or thicken it? At the low wind speeds of a street canyon, does millimetre roughness raise the convective coefficient, or trap a deeper stagnant film? This single question decides whether B works at all. [TODO: Check by measurement: two identical plates, one sprayed, logged over a hot afternoon.]

### Engineering check 1: does the loop drive itself?

A thermosiphon circulates because the fluid in the warm leg is less dense than in the cool leg: the warm riser weighs less than the cool return, and that weight difference is the pump. The available driving pressure over a loop of height h is

Δp  =  β · ΔT · ρ · g · h

| Symbol | Meaning | Unit | Value used | Source |
| --- | --- | --- | --- | --- |
| Δp | driving pressure created by the density difference | Pa | result ≈ 470 | — |
| β | volumetric thermal expansion coefficient of water | 1/K | 3·10⁻⁴ | property, water at ~30 °C |
| ΔT | temperature difference between riser and return | K | 10 | assumed, not measured |
| ρ | density of water | kg/m³ | 995 | property at ~30 °C |
| g | acceleration of gravity | m/s² | 9.81 | constant |
| h | height between the collecting and releasing sections | m | 16 | five storeys plus roof structure, reference site |

Table 3.8 — Assumptions for Engineering check 1.

Result. β·ΔT is the relative density difference, here 0.3 %; multiplied by ρ·g·h it becomes a pressure. The result is about 470 Pa — real, but small, roughly the pressure of a 5 cm column of water.

How much water has to move follows from Q̇ = ṁ · c_p · ΔT. With an absorbed flux of 100 W/m² over one 288 m² façade the loop carries about 29 kW, which at ΔT = 10 K needs a mass flow of about 0.7 kg/s. Pushing that through a pipe costs friction, and only 470 Pa are available: a bore of 50 mm fails, a bore of about 80 mm passes with margin for bends and the plate.

What this does and does not prove. It establishes that the concept does not need a pump, which is what the energy constraint demands, and it fixes a parameter that was previously free: the risers must be of the order of 80 mm, not the 15 mm of ordinary heating pipework. That is an architectural consequence — it sets the build-up thickness against the soft constraint in Section 2, and it is the reason Concept A is a new-build system. It does not prove that the flow distributes evenly across parallel risers, and it says nothing about winter. [TODO: Repeat for ΔT = 3 K, the weakest driving condition — the head falls to about 140 Pa — and report whether the loop stalls.]

### Engineering check 2: where does the heat go?

The loop can only release what its releasing surface can pass. For a surface close to ambient the net release at night is of the order of 80–100 W/m², combining long-wave emission to a clear sky with free convection (Raman et al., 2014; Zhai et al., 2017). The required area follows directly:

A_release  =  Q̇ / q″

Q̇ is the whole load, 29 kW; q″ is the same quantity per square metre, about 100 W/m². The double prime means “per unit area”. The result is about 290 m² of release surface.

Result, and what it changed. The roof of the reference block is about 18 × 14 m = 252 m². The roof is therefore too small to release what one façade collects, before any allowance for plant, access or photovoltaics. This is the most useful result in the section, because it moved the problem: the binding limit on Concept A is not the driving pressure — check 1 passes comfortably — but the area available to give the heat away. Section 3.3 carries this into the selection, and it is the weakness Concept V2 answers. [TODO: Recompute q″ for our own sky and emittance assumptions and report a range; and state how much area photovoltaics would claim.]

## Concept Evaluation & Iteration

The two concepts are compared against the six criteria of Section 2, using the two engineering checks as evidence rather than as decoration. Where a rating rests on an estimate rather than a computed number, the evidence column says so.

| Criterion | Wt | A | B | Evidence used |
| --- | --- | --- | --- | --- |
| 1 Cooling slope | [TODO: ] | 5 | 3 | A: the driving head grows with ΔT, so the transport rate itself follows the load (check 1). B: the convective flux is linear in the surface–air difference — positive, but not steepening |
| 2 Reflectance | [TODO: ] | 4 | 4 | Both keep a bright finish; neither has an advantage |
| 3 Surface cooling | [TODO: ] | 5 | 3 | A removes absorbed heat from the wall instead of only reflecting it. B raises the convective term only |
| 4 Street-level comfort | [TODO: ] | 5 | 2 | A releases at roof level, away from head height. B releases into the canyon, where the people are — a real disadvantage the criterion exposes |
| 5 Night release | [TODO: ] | 4 | 2 | A: the loop runs after sunset and the roof plate sees a clear sky. B: no store, so little is left to release |
| 6 Staying power | [TODO: ] | 2 | 5 | A: a buried water loop cannot be renewed without opening the façade. B: sprayed on, sprayed again — the strongest single argument for B |
| Feasibility | [TODO: ] | 2 | 5 | A needs new construction, 80 mm bores and freeze protection. B needs a spray gun |
| Biomimetic transfer | [TODO: ] | 5 | 4 | A transfers an arrangement and a control principle; B transfers a length scale |
| Evidence and uncertainty | [TODO: ] | 3 | 1 | A has two checks with numbers. B rests on an untested assumption about the boundary layer |
| Weighted total |  | [TODO: ] | [TODO: ] | [TODO: fill once the weights are agreed] |

Table 3.9 — Decision matrix. Ratings 1–5. The last three rows are not criteria and are deliberately kept apart from them.

The last three rows carry the questions a criterion table cannot answer: whether the concept can be built, whether it genuinely transfers the biology or merely resembles it, and how well supported the ratings above are. The third is the one that prevents a confident-looking table from concealing guesswork, and it is where B scores worst: its decisive assumption has not been tested.

What the comparison shows. A leads on five of six criteria and B on one — but the one it leads on is durability, which is exactly where A is weakest, and B is far ahead on feasibility. Two concepts whose strengths are complementary rather than overlapping are an argument for a hybrid, not for a winner.

### Review and feedback

[TODO: World café peer feedback, instructor feedback, and AI used as a critical reviewer. For every comment record the decision — accept, reject or investigate — and the justification. A documented rejection is worth more than a list of accepted comments. This belongs in Appendix A as well.]

### Selection and Final Concept V2

Selected: a hybrid, A with B applied to it. Neither concept is carried forward alone. Engineering check 2 showed that the roof cannot release what one façade collects — about 290 m² required against 252 m² available — so Concept A as first drawn is limited not by its physics but by the area of its one releasing surface. Concept B raises the release rate of any surface it is sprayed on, at negligible cost and with no construction.

What V2 changes. The roof plate stops being the only radiator. In V2 the ribbed façade panels are part of the return path, so the façade itself releases at night what it collected by day, and the roughness spray is applied to both the façade panels and the roof plate to raise the convective coefficient on each. The releasing area is then the façade plus the roof rather than the roof alone, which is what closes the gap check 2 opened. A second consequence follows without being designed for: the spray is renewable, so the part of the system that wears is also the part that can be replaced — which repairs A's weakest criterion.

| What changed | Why it changed | Evidence that informed the change |
| --- | --- | --- |
| The façade became a radiator as well as a collector | The single roof plate could not pass the collected flux | Engineering check 2: ≈ 290 m² required, 252 m² available |
| The roughness spray was applied to A rather than kept as a rival | It raises the release rate of every surface it touches, and it is renewable | Decision matrix: B leads criterion 6, the one criterion on which A scores 2 |
| The switching coating was dropped before development | It has no effect in the night window | Section 2: −0.1 K at night is the incumbent's failure, not something to reproduce |

Table 3.10 — Final output of the creating and evaluating phase: Concept V1a/b → engineering review → evidence → Concept V2.

### Remaining uncertainties

Four are known and none is closed.

Does the texture pierce or thicken the boundary layer? B's decisive assumption is untested, and V2 now depends on it in two places rather than one.

Does the loop survive a Swiss winter? The climate constraint requires operation to about −10 °C; freeze protection without auxiliary energy is not designed.

Does the loop still drive itself at small ΔT? Check 1 was run at 10 K; at 3 K the driving head falls to about 140 Pa and the margin may vanish.

Does millimetre texture survive twenty years? It sits on the outside of the building, and criterion 6 asks for 90 % retention after three.

[TODO: Rank these by how much each would change the design if it failed, and say which one you would test first and how.]

# Sustainability & Life’s Principles

## What this section assesses, and what it does not claim

Section 2.8 named the principles this project works under. Naming them is not applying them, and the difference is the whole content of this section: each principle below is stated as a claim, followed by the evidence for it and by the point at which the concept falls short of it. A principle with no shortfall named is a principle that has not been assessed.

Two limits are stated at the outset rather than buried. First, the assessment of the final concept can only be as specific as the concept, and Section 3.3 has not yet fixed it; the parts of this section that depend on that choice are marked. Second, no life cycle assessment was carried out. Where embodied emissions are discussed they are discussed as a criterion with a decision rule, not as a computed result, and the words “sustainable” and “resource efficient” are avoided as descriptions of the concept because on the present evidence neither could be defended.

## Life’s Principles applied to the concept

| Principle and sub-principle claimed | How the concept applies it | Where it falls short |
| --- | --- | --- |
| Be locally attuned and responsive — the signal, the antenna and the response match | F9 is this principle expressed as a function: the surface temperature is the signal, the physics of the removal path is the antenna, and the cooling power is the response. Both sinks used — the night sky and rainwater the city currently drains away — are local and freely available. | The response is attuned to temperature, not to the quantity that actually matters to people, which is radiant load at eye level. The concept optimises what it can sense, not what it is for. [TODO: Revisit once V2 is fixed.] |
| Be resource efficient — use low energy processes; one design meets more than one need | The cooling function consumes no auxiliary energy; this is a hard constraint, not an aspiration. Multi-functionality is set as an explicit tie-breaker for Section 3.3: a build-up in which one layer carries several functions is preferred to one that needs a layer per function. | Nothing has yet been counted. “Resource efficient” is a claim we cannot make until the embodied emissions of the added build-up are estimated against the cooling it delivers over its service life. |
| Adapt to changing conditions — adapt to temporal and spatial changes; maintain integrity through self-renewal | The design case is a five-day heat wave with a reset condition, not one hot afternoon. Durability against soiling is a criterion in its own right (KPI 7) rather than an afterthought. | The seasonal half of the principle is weakly served. A surface optimised for July is a surface that also faces January, and the winter trade-off is currently a constraint we accept rather than a behaviour we design. |
| Use life-friendly chemistry — safe subset; harmless breakdown products | The decision that self-cleaning must be physical rather than chemical follows directly from this principle, and it is a design constraint, not a preference: anything that leaches from the surface enters rainwater which the concept then evaporates at pedestrian level. | The principle is currently satisfied by exclusion — we have ruled substances out. No substance we intend to use has yet been checked in a toxicity database. See Section 4.7. |
| Integrate development with growth — built to shape | A modular element sized to a façade grid produces no offcuts. On-site material waste in construction is put at 10–15 % by the Ellen MacArthur Foundation (Ellen MacArthur Foundation, 2015), so this is a measurable quantity rather than a gesture. | Claimed narrowly and on purpose. The sub-principle creates more opportunities for life is not claimed: a permanently damp textured surface invites exactly the algal growth KPI 7 penalises. |
| Evolve to survive | Not claimed for the artefact. A façade layer does not evolve across generations. It is claimed for the process: this report builds on the incumbent measure rather than starting from zero, and documents what was de-selected and why. | — |

Table 4.1 — Life’s Principles assessed against the concept. The last column is the part that matters: a principle claimed without a shortfall has not been assessed.

## Eco-efficient or eco-effective?

Cradle to Cradle draws a distinction this project has to face directly. An eco-efficient design is less bad: it does the same thing while emitting less. An eco-effective design is actively good: its outputs are inputs somewhere else. The standard illustration is a brake pad — one version emits fewer particles but still emits thousands of tonnes, the other is made of a material that can safely re-enter a biological cycle. Nature, the argument runs, is not efficient at all: a cherry tree produces blossom wastefully, and is nonetheless effective because every part of that waste is a nutrient.

Measured against that distinction, Concept V2 is eco-efficient and no more. A façade that absorbs less, moves what it does absorb to a better surface and gives it away without energy is a less bad façade; it produces no nutrient for anything.

One thing is worth stating precisely, because it is easy to overclaim. The loop is closed: no water is consumed, none evaporates, and nothing is drawn from the municipal supply. Against an evaporative façade — the obvious alternative, and one already built as HydroCanopy (AskNature, n.d.) — that is a genuine advantage under the water constraint, and it is the honest way to state it: V2 avoids a resource cost rather than creating a resource benefit. Avoiding a cost is efficiency. It is not effectiveness.

An eco-effective version would have to make its output the input of something else. Two routes exist and both were placed outside the boundary in Section 2: recovering the collected heat for domestic hot water, which the boundary excludes as “reuse of the stored energy”; and harvesting the rainwater that falls on the same roof plate. Neither is pursued here, so the project remains at the efficient end by a boundary decision rather than by a physical limit. That is the first thing a follow-on project should revisit.

## The material perspective: which layer belongs in which cycle

Cradle to Cradle divides materials into biological nutrients, which are consumed and must return safely to the environment, and technical nutrients, which are used without being consumed and are recovered (McDonough & Braungart, 2002). The rule that assigns a material to one or the other is about wear: anything that is consumed — worn out — has to come from the biological cycle, and anything that is merely used, without wear, can come from the technical nutrient cycle. Applied layer by layer, this decides the material strategy of the build-up before any material has been chosen, and it produces one uncomfortable result.

| Layer | Worn or used? | Consequence |
| --- | --- | --- |
| Outer finish or coating | Worn — UV, driving rain, freeze–thaw, soiling | Must be biologically benign in breakdown. It is abraded into the street, and into the run-off we collect |
| Water-carrying absorbent layer | Worn, and in direct contact with the run-off | Same requirement, for the same reason |
| Ventilated cavity, sub-structure, fixings | Used, not consumed | Technical cycle: recoverable, demountable, and worth specifying as separable single materials |
| Masonry behind | Untouched by the retrofit | Outside the product |

Table 4.2 — The wear rule applied to the build-up.

The uncomfortable result. The layer we would most readily engineer with chemistry — the outer finish, where the optical performance lives — is precisely the layer that must be biologically benign, because it is the one that wears away into the place we are trying to improve. That inverts the usual order of a coating specification, in which durability and optical performance are chosen first and the environmental profile is checked afterwards.

## Circular design principles: an honest status

| # | Principle | Status | Evidence or reason |
| --- | --- | --- | --- |
| 1 | No ecotoxic chemicals | Answered | Ruled out by constraint; the self-cleaning function must be physical (Section 4.7) |
| 2 | Recyclable materials | Depends on V2 | [TODO: state the material of each layer and whether it is separable] |
| 3 | Easy to dismantle, modular | Partly answered | Built to shape and demountable is a stated design rule; the fixing detail is not designed |
| 4 | Durability | Answered | KPI 7: ≥ 90 % of the cooling function retained after three years; service life target 20 years |
| 5 | Maintainability and reparability | Partly answered | Soft constraint “functional with little maintenance”; no repair concept exists |
| 6 | Cascade use of raw materials | Not addressed | Nothing in the concept anticipates a second life for its materials |
| 7 | Planned return logistics | Out of boundary | Requires a business model; Table 2.7 places this outside the project |
| 8 | Efficient use of energy | Answered | Hard constraint: no auxiliary energy for the cooling function |

Table 4.3 — The eight cycle design principles, with the status of each. Three can be answered today, three depend on the final concept, and two lie outside the project boundary.

Items 6 and 7 are the honest weaknesses of this project from a circular perspective, and they are related. Both need somebody to own the material after its first life, which in turn needs a business model — rental, service or deposit — rather than a sale. The reason is structural and is worth stating because it is not our oversight: in a sales model the manufacturer transfers ownership, so the value created by durable, recoverable design accrues to the recycler, the user and the maintenance trade, and not to the party who paid for it. That split incentive is named as the principal obstacle to circular construction (Ellen MacArthur Foundation, 2015), and a façade retrofit has it in an acute form: the owner pays, the tenant gains comfort, the street gains the cooling, and nobody owns the outcome.

## Environmental benefits, limitations and trade-offs

Frost protection against life-friendly chemistry. This is the trade-off V2 created, and it has no comfortable answer. The loop must survive to about −10 °C without auxiliary energy. Glycol is the standard solution and would put a mobile additive inside a pipe running the full height of an occupied façade, where the failure mode is a leak into the wall or the ground. That collides directly with use life-friendly chemistry. Draining for winter avoids the substance but stops the system working in the shoulder seasons and adds an operation somebody must remember. A third route — sizing and routing the loop so that it is self-draining by gravity — keeps the chemistry clean and is the one worth checking first, because it is the only option that does not trade a principle for a season. [TODO: Establish which of the three is feasible; if an additive is unavoidable, name it and check it in a toxicity database.]

Summer against winter. A high solar reflectance reduces summer heat gain and also reduces useful passive solar gains in winter. The constraint in Table 2.3 requires that heating demand must not appreciably rise. [TODO: Estimate both halves for the Swiss climate and give the net annual balance. Until that number exists this is a known unquantified risk, not a solved trade-off.]

Cooling against water. Evaporative cooling costs 2.45 MJ per kilogram of water evaporated. For an indicative 100 W/m² over the seven-hour load window this is of the order of one litre per square metre and day, so roughly five litres per square metre across the five-day design case — which is within what a 20 mm absorbent layer can hold, and therefore not the binding limit. [TODO: Recompute with our own assumptions and report a range rather than a point.] The binding limits are metering, frost, and the fact that during a heat wave it does not rain: the reserve must be charged beforehand.

Local against global. The concept moves heat out of the street canyon. Whether it reduces the total energy in the system or merely redistributes it is a different question, and the honest answer differs by mechanism: a radiative path sends energy to space and is a genuine removal, while a convective or evaporative path relocates it within the urban boundary layer. [TODO: State which of these the final concept relies on, and do not claim a reduction where the mechanism only redistributes.]

Materials. [TODO: Pigments, binders, any biocide, the separability of the layers at end of life, and the embodied greenhouse gas of the added build-up against the constraint in Table 2.3.]

## Circular does not mean sustainable

Keeping a material in circulation is not by itself an environmental benefit. The standard counter-example is an additive that is harmless in its first application, in a laptop housing, and hazardous in its second after recycling into a children’s toy: the substance did not change, the exposure pathway did.

Our project has the same failure mode one step earlier, and it is the reason the life-friendly-chemistry principle is not decorative here. A biocide or a stabiliser that is entirely acceptable in a façade coating becomes unacceptable once it leaves that coating, because the path it leaves by is the rainwater this concept deliberately collects and then evaporates at pedestrian level. A substance that would be diluted into a sewer in the incumbent case is, in our case, delivered to the air people breathe. The concept concentrates its own emissions where its beneficiaries are.

Two consequences follow. The self-cleaning function must be structural rather than chemical — which the lichen strategy of Section 3.1 shows is physically possible and the commercial lotus-effect coating cited in Section 2.3 shows is manufacturable (Sto SE & Co. KGaA, 1999). And any substance that does remain in the specification must be checked rather than assumed.

[TODO: Do this check and report it: name the substance, the database, and the result. The REACH registry at the European Chemicals Agency ⟦(European Chemicals Agency, n.d.)⟧ is the primary source; PubChem and the EPA DSSTox database are alternatives. One short paragraph naming a substance, a database and a finding is worth more here than a page of general language.]

## Did sustainability considerations change the design?

The template asks this directly, and it is the question a sustainability section most easily fails, because the honest answer is often no. Here it is yes, in three documented instances, each traceable to a decision recorded in the decision log in Appendix B.

1. The reflectance target was lowered from what is technically possible. Ultra-white coatings reach a solar reflectance of about 0.98 (Li et al., 2021), and an earlier version of this project treated that as the goal. KPI 2 now targets ≥ 0.80, the city’s own threshold, because the principle of optimising rather than maximising makes the further reflectance a cost in glare, winter gains and photovoltaic yield rather than a benefit. The performance target was changed by a sustainability argument.

2. The self-cleaning function was constrained to a physical mechanism. The obvious solution to soiling is a biocide in the film. The leaching argument above removed that option before any concept was developed, which is why the lichen strategy entered the model table at all.

3. Cost and embodied emissions were reclassified. They were originally an eighth KPI. Recognising that a KPI must follow from a function moved them into the constraints of Table 2.3 — where a violation disqualifies a concept rather than merely scoring against it. The change made the requirement stricter, not weaker.

## Limits of this assessment

No life cycle assessment was performed, so no statement in this report should be read as a quantified environmental balance. The embodied emissions of the added build-up are a constraint with a decision rule and not a measured figure. The assessment of Life’s Principles above is made against a concept direction rather than a finished design, and [TODO: must be repeated against Concept V2 once Section 3.3 selects it]. Finally, two of the eight circular design principles — cascade use and return logistics — are not addressed at all, and the reason is a boundary decision rather than an analysis: they require a business model, and this project does not design one.

# Discussion & Conclusion

## Contribution of the biomimicry process

[TODO: Be concrete and honest. Which idea came specifically from the biological model, and which would a conventional engineering approach have produced anyway? A well-argued “this part was conventional” is worth more than an overstated claim.]

## Strengths and weaknesses of the final concept

[TODO: ]

## Assessment against the challenge and the KPIs

[TODO: Go back through KPIs 1–7 from Table 2.9 and state for each one whether it was met, missed, or could not be assessed with the available evidence.]

## Innovative character and limits of the evidence

[TODO: State clearly what has been demonstrated (calculation, measurement, literature) and what remains an assumption.]

# References

[1]	L. Yang, F. Qian, D.-X. Song, und K.-J. Zheng, «Research on Urban Heat-Island Effect», Procedia Eng., Bd. 169, S. 11–18, 2016, doi: 10.1016/j.proeng.2016.10.002.

# Glossary

U

- UHI – Urban Heat Island

- UHII – Urban Heat Island Index

“Picture”

| Students | Contribution to the report / (sections) | Comments |
| --- | --- | --- |

Remarks:

• In the report you should outline and document your research and design approach. All chapters must be addressed, although emphasis may differ depending on the challenge.

• There is no fixed length. As a guideline, 15–25 pages (excluding references and appendices) are appropriate. Use figures, functional diagrams, annotated concept drawings, tables and calculations where useful.

• Use the headings from this template.

• Refer to all sources used, including AI-supported tools. AI output is not evidence and relevant biological and technical claims must be verified.

# Appendix A – AI Use Statement

Relevant AI-supported activities are documented below. Not every prompt is listed; the focus is on how AI contributed to the work and how important outputs were verified. AI output is not used as evidence for factual claims.

| AI tool | Used for | How the output influenced the project | How important output was verified |
| --- | --- | --- | --- |
| AskNature (search and strategy pages) | Discovering biological strategies for functions F1–F12, navigated by taxonomy branch rather than by organism | Produced the candidate list in Table 3.2, including four models that were subsequently de-selected | Every mechanism claim carried into the report is referenced. The pages could not be opened programmatically, so titles and URLs were taken from search results; the open item in Section 3.1 records that each must be confirmed and, where a primary paper exists, replaced by it |
| Large language model assistant | Structuring the scoping, distilling the course material into working notes, drafting report text, building the transient wall model, and generating prompts for figures | Draft text and table structures throughout Sections 1–4; the search-path table; the transient model and its figures | No factual claim entered the report without a named source. Benchmark values were checked against the Fachplanung. Two errors in the AI-built wall model were found and corrected: an air-cavity node made the explicit scheme unstable, and the synthetic weather drive had an inverted diurnal phase, which showed up as a peak surface temperature exactly equal to the air temperature |
| AI as critical reviewer | [TODO: Section 3.3 asks for this. Give the reviewer the concept sketch, the Engineering Canvas, the functions and KPIs, and the assumptions] | [TODO: Record each comment and the team’s decision: accept, reject or investigate] | [TODO: A documented reject with a reason is worth more here than a list of accepted comments, because it shows judgement] |
| [TODO: Further tools used by the team] | [TODO: ] | [TODO: ] | [TODO: ] |

Table A.1 — AI use statement.

On the limits of this statement. Nothing in this report is cited to an AI tool. Where an AI-assisted search surfaced a candidate, the candidate was carried forward only with a source of its own, and the two remaining places where that chain is incomplete are marked as open items rather than left implicit: the AskNature pages listed above, and the 23 % figure for elephant hair, which traces to a university news article rather than to a study (University of Notre Dame, 2021).

# Appendix B – Decision Log

Every decision below is argued at the place named in the last column. This table exists so that the reasoning can be checked without re-reading the report, and so that what was not chosen stays visible: a design process is only legible if the discarded alternatives are on the record.

| Decision | Chosen | Rejected alternative | Reason, and where it is argued |
| --- | --- | --- | --- |
| Design question | Let the absorbed heat drive its own removal, self-regulating | Four other candidates, from “prevent heat build-up in cities” to “turn heat into a resource” | Too broad, unanchored, factually wrong, or not verifiable in the time available. The chosen question adds the self-regulating clause, which is the actual content of the project. Section 2.1 |
| Reference city | Zurich | A generic European city | An adopted municipal heat strategy, published target values, open geodata and long-term stations make the targets checkable instead of invented. Section 2.2 |
| Reference site | Modellierungsgebiet 03, closed perimeter block | A freely chosen building | The city publishes modelled baseline and climate-optimised results for exactly this area, so the concept is compared against a documented reference. Section 2.2 |
| Design case | Five consecutive hot days | One hot afternoon | Only a multi-day case stresses the water reserve and the reset behaviour of the store; a concept that works on day one and saturates by day three has not solved the problem. Section 2.2 |
| Evening window | 18:00–24:00 | 20:00–24:00 | A window must contain the instant at which its own KPI is evaluated, and the two hours before 20:00 are among the hottest of the day at the surface. Section 2.2 |
| System of interest | The façade, with the roof as the release interface | The envelope as a whole, roof included | The roof receives more than twice the noon irradiance, but releases its heat upward to a sky it can see; the façade releases sideways into the canyon at head height, and Section 1 fixes that as where the effect must appear. Section 2.2 |
| Number of systems | One | Two — “reduce heat” and “prevent heat” | The systems view showed these are two functions of one system, coupled by the design question. Section 2.4 |
| Scope of “use the heat” | Let it drive its own removal | Seasonal storage and electricity generation | Conversion competes with photovoltaics for the same surface and any seasonal store lies outside the envelope. Section 2.4 |
| Selection KPI for the urban effect | PET at 2 m above ground | Urban heat island intensity | A district-scale quantity that a single retrofit cannot move measurably, and it invites the surface-versus-air confusion. Section 2.7 |
| Cost and embodied emissions | Constraints | An eighth KPI | A KPI must follow from a function; these are conditions to satisfy, not measures of performance. The move also brings the list inside the required four to seven. Section 2.7 |
| Albedo target | ≥ 0.80, the city’s threshold | The technical maximum of ≈ 0.98 | Life’s Principles ask for optimised rather than maximised; further reflectance costs glare, winter gains and photovoltaic yield. Section 2.8 |
| Life’s Principles claimed | Four claimed with the sub-principle named | “Creates opportunities for life” and “Evolve to survive” for the artefact | A damp textured surface invites the algal growth KPI 7 penalises, and a façade layer does not evolve across generations. Naming the conflict is more honest than claiming the principle. Section 2.8 |
| Search strategy | By function, through the taxonomy | By organism | Searching for organisms would not have produced F9, which is the pairing of two taxonomy branches rather than a single one. Section 3.1 |
| Termite mound | De-selected | The standard biomimicry reference for passive cooling | The chimney account is contested in the literature, and the building usually cited for it uses fans. Section 3.1 |
| Fog harvesting | De-selected | The Namib beetle, the most-cited water model | Fog in Zurich is a winter phenomenon; a heat wave brings neither rain nor fog. Section 3.1 |
| Green façades | Out of scope | An obvious and proven measure | Already the client’s HA 09 and HA 10, and in direct competition with our own water reserve, which is scarcest exactly when planting needs most. Section 3.1 |
| Evidence standard | Primary papers where they exist, the publishing organisation otherwise | AskNature entries treated as evidence in their own right | AskNature and AI-assisted search find candidates. Twelve of twenty model entries still rest on an AskNature page alone; this is recorded as a shortfall rather than presented as met. Section 3.1 |
| Concept pair | [TODO: decide] | [TODO: the two directions not developed] | [TODO: state the physical distinction that makes the pair meaningful] |
| Final concept | [TODO: A, B or a justified hybrid] | [TODO: ] | [TODO: name the weakness in V1 that V2 answers] |

Table B.1 — Decision log. What was chosen, what it displaced, and why.

# Appendix C – Optional Supporting Material

Additional calculations, extended concept drawings, the transient wall model and its figures, and prompt excerpts belong here. [TODO: Add the wall-model results and the measured weather drive used for them.]

# Rubric for grading the Biomimicry Reports (70% of the final grade)

The rubric emphasizes the quality of the bio-inspired transfer and the engineering development of the concept. A visually attractive idea alone is not sufficient for a high grade.

| Criteria | Excellent (4 pts) | Good (3 pts) | Satisfactory (2 pts) | Needs Improvement (1 pt) |
| --- | --- | --- | --- | --- |
| 1. Challenge, Scoping & Functional Analysis 15% | Problem and design question are precise; system boundaries, context and constraints are clear; functions are well formulated; functional diagram is insightful; 4–7 meaningful performance criteria are defined. | Clear problem and functions; system/context mostly clear; functional diagram and performance criteria are useful but show minor gaps. | Basic problem framing; functions or system boundaries partly unclear; functional diagram superficial; performance criteria generic. | Problem framing unclear; important functions/context missing; no meaningful functional diagram or performance criteria. |
| 2. Biological Discovery & Abstraction 20% | Strong research path; relevant natural models compared and selection/deselection justified; mechanisms explained in depth with scientific evidence and figures; convincing abstraction from biology to transferable principles. | Good natural models and mostly correct mechanisms; research path and abstraction clear but less detailed or critical. | Limited research depth; mechanisms remain descriptive; selection logic or abstraction is weak. | Poor or missing biological models; mechanisms unsupported or misunderstood; no meaningful abstraction. |
| 3. Engineering Concept Development 25% | Clear engineering logic and Engineering Canvas; biological principles are convincingly translated into technical principles; at least two technically distinct, well-developed concepts with annotated engineering sketches; components, flows, materials and design decisions are explained. | Engineering logic is mostly clear; two meaningful concepts are developed and annotated; biological-to-technical transfer is understandable but some engineering aspects remain underdeveloped. | Concepts remain largely at idea/sketch level; functional logic or biological-to-technical transfer only partly explained; limited technical detail. | Concepts are superficial or primarily visual; little engineering development; biological inspiration is copied literally or technical transfer is unclear. |
| 4. Engineering Validation & Iteration 15% | 2–3 critical engineering questions are derived from the concept; relevant assumptions are tested with appropriate calculations, estimates, experiments, models or other evidence; results influence concept selection and lead to a clearly documented improved Concept V2. | Critical questions and relevant checks are present; results are interpreted and some concept improvement is demonstrated. | Some technical checking is attempted, but questions are generic or calculations/evidence have little influence on the design. | No meaningful engineering questions or validation; functionality claims remain unsupported; no iteration demonstrated. |
| 5. Sustainability & Life’s Principles 10% | Relevant Life’s Principles are applied critically; environmental benefits, limitations and trade-offs are discussed; circularity/resource aspects are integrated where relevant and linked to design decisions. | Relevant sustainability aspects and Life’s Principles are discussed with reasonable connection to the concept. | Sustainability discussion is generic; Life’s Principles are listed rather than applied; trade-offs receive little attention. | Sustainability claims are unsupported or largely absent; no meaningful application of Life’s Principles. |
| 6. Scientific Quality & Critical AI Use 10% | High-quality and varied sources; consistent citations; biological and technical claims are evidence-based; AI use is transparent and critical; important AI outputs, calculations and claims are independently checked. | Adequate sources and mostly consistent citations; AI use declared; most important claims are verified. | Weak or repetitive sources; inconsistent citations; AI use insufficiently documented or outputs accepted with limited verification. | Major citation problems; unsupported claims/plagiarism; AI-generated content used uncritically or not declared. |
| 7. Communication & Report Quality 5% | Professional, concise and coherent report; figures, diagrams, calculations and tables are well integrated and readable; reasoning chain is easy to follow. | Clear report with useful visuals and generally coherent structure; minor presentation weaknesses. | Some unclear writing or inconsistent structure; visuals are weakly integrated or difficult to interpret. | Hard to follow; poor formatting; key figures/diagrams missing or unclear. |

Scoring note:

Each criterion is rated from 1–4 and multiplied by its weight. The weighted score is converted to the Swiss grade according to the course grading scheme. The report grade contributes 70% of the final course grade.

| Weighted Rubric Score | Swiss Grade | Interpretation |
| --- | --- | --- |
| 1.00 | 3.0 | Needs Improvement |
| 1.25 | 3.5 |  |
| 1.50 | 3.5 |  |
| 1.75 | 4.0 |  |
| 2.00 | 4.0 | Satisfactory |
| 2.25 | 4.5 |  |
| 2.50 | 4.5 |  |
| 2.75 | 5.0 |  |
| 3.00 | 5.0 | Good |
| 3.25 | 5.5 |  |
| 3.50 | 5.5 |  |
| 3.75 | 6.0 |  |
| 4.00 | 6.0 | Excellent |

If you feel a team member should be awarded a higher or lower mark due to his/her contributions to an assignment, you can hand in a statement indicating the weighting of the individual marks. (For example, if the team mark is a 5 for a total of two team members, a weighting of 40% vs 60% will result in the marks 4 and 6, respectively.) This statement must be signed by all team members and handed in on the due date of the assignment.

# Structure & Content

## Introduction

• State the background, relevance and aim of the report clearly.

• Briefly introduce the design challenge and the intended contribution of the project.

## Challenge & Scoping

• Define the problem to be solved precisely and formulate a clear design question.

• Define context conditions and constraints under which the solution must work.

• Describe the system of interest, relevant subsystems, supersystem and system boundaries where relevant to your project.

• Identify the key functions. Functions should be written as verbs and aligned with the taxonomy explainer.

• Provide a first abstract functional diagram showing the main functions and, where relevant, energy, material and information flows. Keep it solution-neutral.

• Formulate biologized questions that guide the search for natural models.

• Define 4–7 performance criteria / KPIs including life principles that will later be used to compare and evaluate concepts.

## 3. Bio-Inspired Design Process

Document the research and design path explicitly. The report should make the reasoning chain from challenge to biological mechanism, technical concept, validation and iteration visible.

### 3.1 Discovering & Abstracting

• Use AskNature / AskNature Chat, scientific literature and other credible sources to identify natural models.

• Make the research path explicit: how were models found, which were selected, and why were others de-selected?

• Describe the relevant biological strategies and mechanisms in depth. Explain how they work rather than only describing the organism.

• Use figures or drawings to support the explanation.

• Abstract the biological mechanism into a general transferable principle. Clearly distinguish biological observation, abstraction and possible technical interpretation.

• Identify relevant Life’s Principles evident in the selected biological strategiyes.

### 3.2 Creating: Engineering Concept Development

• Generate alternative solutions using a structured ideation method such as Crazy 8s and/or a Morphological Matrix. Develop at least three initial concept combinations before selecting two for further development.

• Develop at least two technically distinct concept variants. Annotated concept drawings should show key components, functions, relevant flows, material ideas and important technical parameters.

• Provide an Engineering Canvas and derive 2–3 critical engineering questions: What must be true for the concept to work? Focus on assumptions that could change the design decision.

• Carry out at least 1–2 appropriate engineering checks, including a quantitative estimate or calculation where reasonably possible. Depending on the challenge, additional checks may include simple experiments, models or material/technical comparisons.

• For calculations, show formula, assumptions, units, result and what the result means for the concept.

• Explain how the concepts were generated and how biological inspiration was balanced with technical feasibility. Avoid literal copying of biological forms; focus on transferable functional principles.

### 3.3 Concept Evaluation & Iteration

• Compare the 2 concept variants against the performance criteria / KPIs defined in Section 2. Use a transparent comparison table or decision matrix and justify important ratings.

• Use the engineering checks and critical questions as evidence in the comparison.

• Use peer review (in the world café presentation or before), instructor feedback and/or AI as a critical reviewer to identify weak assumptions and open questions. AI feedback must be assessed critically by the team.

• Select Concept A, Concept B or a justified hybrid solution (in this final report).

• Develop a revised Final Concept V2 and show explicitly what changed compared with the earlier concept and why. Explain the concept in detail

FINAL OUTPUT OF THE CREATING & EVALUATING PHASE

Concept V1a/b → Engineering Review → Evidence → Concept V2 Clearly show what changed, why it changed and what evidence informed the change.

• Discuss remaining uncertainties and identify what should be tested or developed next.

## 4. Sustainability & Life’s Principles

• Life’s Principles most relevant to your final concept and assess how well the concept actually applies them.

• Discuss environmental benefits as well as limitations and trade-offs. Avoid unsupported claims such as “sustainable” or “resource efficient”.

• Where relevant, discuss material use, durability, repair, reuse, modularity, recyclability and/or Cradle-to-Cradle / Circular Economy aspects.

• Explain whether sustainability considerations led to changes in the design.

## 5. Discussion & Conclusion

• Discuss whether and how the biomimicry process contributed to ideas that would probably not have emerged from a conventional design approach.

• Critically reflect on the strongest and weakest aspects of the final concept.

• Assess whether the original challenge and performance criteria were addressed.

• Discuss the innovative character of the solution and the limits of the available evidence.

## 6. References

• Use a varied set of high-quality sources, including scientific papers, AskNature entries and credible technical sources. Approximately 20 references are a useful guideline, but source quality and relevance matter more than a fixed number.

• Use one consistent citation style throughout the report.

• AI tools are not scientific references for factual claims. Verify relevant biological and technical claims using appropriate sources.

## Appendix A – AI Use Statement

Briefly document relevant AI-supported activities. Do not include every prompt. Focus on how AI contributed to the work and how important outputs were checked.

| AI tool | Used for | How the output influenced the project | How important output was verified |
| --- | --- | --- | --- |
| e.g. AskNature Chat | e.g. discovering biological strategies | e.g. generated alternatives for further research | e.g. AskNature profile + scientific paper |

## Appendix B – Optional Supporting Material

• Additional calculations, extended concept drawings or prompt excerpts may be included where useful.