# Sustainability & Life’s Principles

## Life’s Principles applied to Concept V2

Section 2.7 named three Life’s Principles the solution should reflect: be resource efficient, adapt to changing conditions and use life-friendly chemistry. The three subsections below assess how far Concept V2 of Section 3.3 (Figures 3.5 and 3.6) applies each of them. Each principle is stated as a claim, followed by the evidence from Sections 2 and 3 and by the point where the concept falls short. No life cycle assessment was carried out, so the words “sustainable” and “resource efficient” are not used to describe the concept: on the present evidence neither could be defended.

### Be resource efficient — minimize the use of external resources

How Concept V2 applies it. Cooling needs no auxiliary electricity: circulation is driven by buoyancy alone (Section 3.2.7), and the loop is closed, so no water is consumed in operation (Table 3.10). The evaporating wet layer and the fan-driven cavity of Table 3.5 were dropped for their water and power demand. The existing wall and roof are retained (Section 3.3.3).

Where it falls short. Pumpless operation is not demonstrated: the mains alone use most of the ≈300 Pa buoyancy pressure, the drawn roof coils exceed it by a factor of about 67, and refilling after drainback may need a pump (Section 3.3.5). Material input is uncounted: about 4,620 m of roof pipe, 5.8 m³ of water, the panels and the ribs have no embodied-emission figure set against the cooling they deliver.

### Adapt to changing conditions — day/night and seasonal cycles

How Concept V2 applies it. The daily cycle is used rather than fought: a hotter façade produces a larger density difference and more flow, so cooling rises and falls with the load without a controller (Section 3.2.7), and the roof releases most of its heat to the night sky (Section 3.2.8). The seasonal cycle is met by draining the loop before frost and refilling it in spring (Section 3.3.5).

Where it falls short. Seasonal adaptation is withdrawal, not behavior: the system is idle in the shoulder seasons, depends on somebody draining it, and trapped water remains a freeze–thaw risk. Ribs and reflective roof act the same in winter as in summer; the rib spacing that would balance summer shade against winter sun is named in Section 3.3.4 but not chosen.

### Use life-friendly chemistry — no toxic chemicals or materials

How Concept V2 applies it. The heat-transfer fluid is plain water, and frost protection by drainback instead of glycol keeps a mobile additive out of a pipe that runs the full height of an occupied façade (Section 3.3.5). Reflection and emission are sought through the surface optics of the silver ant [22], not through a functional additive.

Where it falls short. The principle is met by exclusion, not by verification. Three substances remain open and unchecked: the timber treatment of the ribs, the binder and preservatives of the roof coating, and any corrosion inhibitor that a drainback loop admitting air on every cycle may need. Section 4.3 reports the one check made so far.

In sum, V2 applies the first principle by design intent and the third by exclusion; the second holds for the daily cycle only. None of the three is demonstrated yet: the resource claim stands or falls with the circulation check ranked first in Table 3.13.

## Environmental benefits, limitations and trade-offs

Section 4.1 looked inward, at whether V2 follows the principles it was designed to. This section looks outward, at what the concept does to its surroundings: the climate, the street, the roof and the building itself.

### Environmental benefits

Displaced cooling load. The climate benefit is not what V2 uses but what it replaces: every hour of air conditioning it makes unnecessary is avoided emissions and avoided waste heat in the street (Section 2.1).

Heat exported, not moved. Radiation to the night sky carries the collected heat out of the urban air; the other release options of Table 3.5, exchange with the air beside the wall or evaporation, leave it in the street (Section 3.2.8).

A cooler street. The shaded façade radiates less onto the pavement, there is no condenser at pedestrian height, and reflection happens on the roof, where it cannot raise heat stress in the canyon [15].

A reversible intervention. Nothing is demolished; the panels can be removed and the building returned to its previous state, so the retrofit creates no demolition waste and consumes no land.

Nothing emitted in operation. No water, no refrigerant, no additive and no noise leave the system while it runs.

### Limitations

A fair-weather benefit. The export of heat depends on a clear, dry night sky. In the humid, hazy nights of a heat wave most of the heat only warms the roof air, so the concept helps least in the conditions it is meant for (Section 3.2.8).

Idle for most of the year. Outside the summer the drained loop does nothing, and no other environmental function fills the gap.

A benefit without a number. No cooling figure exists, so the avoided emissions cannot be counted, and the whole benefit rests on circulation without a pump, the first open check of Table 3.13.

A balance with one side. The added materials carry embodied emissions that no life cycle assessment has set against the avoided cooling load (Table 2.1).

### Trade-offs

Townscape. Ribs and panels change how the building looks; on a listed façade this collides with heritage protection [19, S. 148–151].

Winter heating. What shades and reflects in summer also rejects winter sun and can raise heating demand and its emissions [23]; the net annual effect is unknown.

Roof area. Every square metre that releases heat is a square metre without photovoltaic yield (Figure 3.4).

Additive or downtime. Avoiding antifreeze costs the shoulder seasons and admits air that may bring back a corrosion inhibitor (Section 3.3.5).

Access or invisibility. Repairable external pipes are the same visible layer that alters the façade.

Under a clear summer sky V2 exports heat from the city at no running cost; the rest of the year it is a visible layer on a listed façade that yields nothing.

## Materials, durability, repair and circularity

Cradle to Cradle separates biological nutrients, which are consumed and must return safely to the environment, from technical nutrients, which are used without wear and recovered [25]. The rule that assigns a layer to a cycle is wear: what is worn out must be able to enter the biological cycle, what is only used can stay in the technical one [26]. Applied layer by layer to the build-up of Figure 3.5, it gives the following picture of what V2 has decided and what is still open.

Timber ribs. Worn by UV, driving rain and freeze–thaw, so they belong to the biological cycle: what weathers off lands on the pavement, and the timber must be untreated, naturally durable, or protected by detailing rather than by chemistry. Species and treatment are open (Section 3.3.4).

Collector panels and finish. Slow surface wear, otherwise used as structure: technical cycle, if the panel is one recoverable material with a benign finish. The panels are designed to be removed (Figure 3.8); their fixings are not detailed.

Roof coating. Worn by UV, rain, soiling and cleaning, and abraded into run-off that reaches the stormwater network of Section 2.3, so it must break down harmlessly. It also carries the assumed reflectance 0.95 and emittance 0.90 (Section 3.2.8). The pigment has been checked (below); binder and preservatives have not.

Pipework, carrier plate, supports and fixings. Used, not consumed: technical cycle, recoverable if specified as separable single materials. At about 4,620 m of roof pipe this is the largest mass in the build-up. The roof pipes under the plate cannot be reached for repair the way the façade pipes can.

Water in the loop. Used, but mobile, and in neither cycle cleanly: about 5.8 m³ is drained to storage each winter and eventually disposed of, and with an inhibitor it is the one component that can leave the system by accident.

Existing wall and roof. Untouched and outside the product. Retaining them is the largest material saving of V2 against a new build.

Measured against the eight cycle design principles of the lecture [26], V2 answers three in principle and none with evidence. Modular construction (3) and reparability (5) follow from moving the pipes out of the wall: a panel can be removed and the façade pipes inspected without opening the wall (Sections 3.3.2 to 3.3.4), but the roof pipes cannot. Efficient use of energy (8) is a hard constraint of Table 2.1 and an open check, not a result. Durability (4) is defined by KPI 6 but has neither a target nor ageing data (Table 3.10). Ecotoxic chemicals (1) and recyclable materials (2) are partly answered, as the layer assessment above shows. Cascade use (6) and return logistics (7) are not addressed: both need an owner of the material after its first life, which is a business model, and Table 2.2 confines the project to the façade and roof.

Circular is not the same as harmless. Keeping a material in circulation is not by itself a benefit: an additive that is harmless in a laptop housing can be hazardous after recycling into a toy, because the exposure pathway changes while the substance does not [26]. V2 has the same failure mode, and every wearing layer has a defined exit: the coating into run-off, the timber treatment onto the pavement, an inhibitor into the wall or the ground on a leak. Durability should therefore come from detailing and material choice before chemistry, and any substance that remains must be checked rather than assumed.

The check made so far. White cool-roof coatings owe their reflectance to titanium dioxide, so it was checked in the REACH database of the European Chemicals Agency [27]. The powder form was classified in 2020 as a suspected carcinogen by inhalation (Regulation (EU) 2020/217, H351); the General Court annulled the classification in 2022 and the Court of Justice confirmed the annulment on 1 August 2025 [28], so the pigment currently carries no harmonized hazard classification. The result is not clean: the concern was dust inhalation, not the applied film, and the ruling turned on the reliability of one study, not on a finding of safety. The pigment is also not the coating: the binder and the film preservatives are what reach the run-off, and they, the timber treatment and any inhibitor remain unchecked. One substance of at least four has been checked; the other three belong to item 5 of Table 3.13.

## Did sustainability considerations change the design?

Yes, in four instances that are documented in Sections 3.2 and 3.3 rather than asserted here.

1. The reflective surface moved from the façade to the roof. Concept C reflected on the façade and was set aside because reflective walls raised pedestrian heat stress in simulations of a Stuttgart street canyon [15]; Concept A reflects on the roof instead (Section 3.2.2). The change was made for the people at street level, not for cooling performance.

2. Water and power demand removed two options before any concept was drawn. The evaporating wet layer and the fan-driven cavity of Table 3.5 were not developed further because of their water and power demand (Section 3.2.2). The Energy and Water constraints of Table 2.1, which are be resource efficient written as constraints, decided this.

3. The pipes moved out of the wall. V1 embedded them in the façade; the engineering review judged hidden damage and the lack of access unacceptable (Table 3.11), and V2 placed them in separable external panels with a drainage gap behind them (Section 3.3.3). Dismantling and repair changed the construction, and the change also opened the concept to existing buildings.

4. Water instead of glycol. Frost protection was decided by drainback rather than by an additive (Section 3.3.5). Use life-friendly chemistry cost the concept its shoulder-season operation, and the trade was made knowingly.

Two considerations did not change the design and should be named as such: the winter balance and the embodied emissions of the build-up are constraints in Table 2.1 that remain unquantified. They have shaped what this report refuses to claim, not what the concept is.

## Limits of this assessment

No life cycle assessment was performed, so nothing in this section is a quantified environmental balance. The assessment is made against Concept V2 as described in Section 3.3, whose central claim of buoyancy-only circulation is the first-ranked open check of Table 3.13; if a pump is needed, the Energy constraint and the resource-efficiency assessment of Section 4.1 fall with it. Durability is defined by KPI 6 but has no data, cascade use and return logistics lie outside the project boundary, and of the substances the concept would put into the street one has been checked and three have not.

## References added by this section (to be entered in Zotero)

[25] M. Braungart und W. McDonough, Cradle to Cradle: Remaking the Way We Make Things. New York: North Point Press, 2002.

[26] ZHAW Blockweek Bionics 2026, «Cradle to Cradle», Vorlesungsfolien (38 Folien), Zürcher Hochschule für Angewandte Wissenschaften, 2026.

[27] European Chemicals Agency, «REACH registered substances database». Zugegriffen: 3. September 2026. [Online]. Verfügbar unter: https://chem.echa.europa.eu/

[28] Gerichtshof der Europäischen Union, Urteil vom 1. August 2025, verb. Rs. C-71/23 P und C-82/23 P, Frankreich und Kommission / CWS Powder Coatings u. a., ECLI:EU:C:2025:601.